无线通信中的人工智能

案例实践报告

案例：	一种基于KNN的调制识别

班级：	20232111XX

学号：	2023210XXX

姓名：	XXX

2025年 5 月 31 日

实验目的

本实验旨在掌握无线通信中基于机器学习的调制识别技术。具体目标包括：理解AMC（自适应调制编码）技术的工作原理，学习kNN算法在信号分类中的应用，以及通过编程实践实现BPSK、QPSK、16QAM三种调制方式的自动识别。

实验内容

本次实验围绕基于kNN的调制识别系统展开。实验要求利用高阶累积量作为信号特征，构建kNN分类器对调制信号进行识别。具体参数设置为：每组数据包含500个调制符号用于特征计算，训练集每种调制方式包含10个样本，测试集每种调制方式包含1000个测试数据，kNN的k值设为10。

实验包含两部分任务：必做部分要求完成编程实现、参数调试及性能分析；选做部分要求将特征从4阶累积量扩展到6阶累积量，对比识别效果的提升。此外还需调整参数进行对比实验，包括将每组符号数从500改为200，以及将k值改为3和50。

实验方法和算法原理

AMC技术原理

AMC（自适应调制编码）技术根据信道质量动态调整调制方式。当信噪比较高时采用高阶调制（如16QAM）以提高频谱效率，当信噪比较低时采用低阶调制（如BPSK）以保证传输可靠性。该技术通过CQI（信道质量指示）反馈机制实现，接收机将信道质量信息通过信令反馈给发射机，发射机据此选择合适的调制编码方案。

kNN算法

kNN（k-Nearest Neighbor）是一种基于实例的监督学习算法。其核心思想是：给定一个待分类样本，计算它与训练集中所有样本的距离，选取距离最近的k个样本，统计这k个样本中出现频率最高的类别作为分类结果。算法的关键参数包括特征提取方法、距离度量方式和k值选择。

高阶累积量特征

高阶累积量是描述信号统计特性的重要工具。对于零均值复随机过程，其高阶矩定义为M_pq = E[X(t)^(p-q) * X*(t)^q]，高阶累积量可由高阶矩推导得到。高阶累积量的优势在于：对于高斯分布的随机变量，其3阶及以上累积量恒为零，因此具有良好的抗噪声干扰能力。

不同调制信号的高阶累积量具有明显差异，这为调制识别提供了特征基础。实验中常用的特征包括4阶累积量（C40, C41, C42）和6阶累积量（C60, C61, C62, C63）。

实验平台

| 组件 | 版本 | 用途 |
| --- | --- | --- |
| macOS | Darwin (arm64) | 操作系统 |
| Python | 3.12.13 | 编程语言 |
| uv | — | 包管理与虚拟环境 |
| matlab | 2024a | 数据生成 |

实验步骤

补全Matlab中关于六阶累积量的代码，随后调整参数，运行Jupyter Notebook，得到结果并整理。

实验代码

六阶累积量生成的代码

function [cumulants] = func_get_cumulants(signalData)

shape = size(signalData);

H = shape(1);

L = shape(2);

cumulants = zeros(H, L);

for row = 1:H

M20 = sum(signalData(row,:).^2)/L;

M21 = sum(abs(signalData(row,:)).^2)/L;

M40 = sum(signalData(row,:).^4)/L;

M41 = sum(abs(signalData(row,:)).^2.*signalData(row,:).^2)/L;

M42 = sum(abs(signalData(row,:)).^4)/L;

% 补充的 Mpq

M60 = sum(signalData(row,:).^6) / L;                             % p=6,q=0 -> X^6

M61 = sum(signalData(row,:).^4 .* abs(signalData(row,:)).^2) / L; % p=6,q=1 -> X^5 X^*

M62 = sum(signalData(row,:).^2 .* abs(signalData(row,:)).^4) / L; % p=6,q=2 -> X^4 (X^*)^2

M63 = sum(abs(signalData(row,:)).^6) / L;                         % p=6,q=3 -> |X|^6

M22 = sum(conj(signalData(row,:)).^2) / L;                        % p=2,q=2 -> (X^*)^2

M43 = sum(signalData(row,:) .* conj(signalData(row,:)).^3) / L;  % p=4,q=3 -> X (X^*)^3

C20 = M20;

C21 = M21;

C40 = M40 - 3*M20^2;

C41 = M41 - 3*M20*M21;

C42 = M42 - abs(M20)^2 - 2*M21^2;

C60 = M60 - 15*M20*M40 + 30*M20^3;

C61 = M61 - 5*M21*M40 - 10*M20*M41 + 30*M21*M20^2;

C62 = M62 - 6*M20*M42 - 8*M21*M41 - M22*M40 + 6*M22*M20^2 + 24*M20*M21^2;

C63 = M63 - 9*M21*M42 + 12*M21^3 -3*M20*M43 - 3*M22*M41 + 18*M20*M21*M22;

C21_modify = C21;

C20_norm = C20/(C21_modify^2);

C21_norm = C21/(C21_modify^2);

C40_norm = C40/(C21_modify^2);

C41_norm = C41/(C21_modify^2);

C42_norm = C42/(C21_modify^2);

C60_norm = C60 / (C21_modify^3);

C61_norm = C61 / (C21_modify^3);

C62_norm = C62 / (C21_modify^3);

C63_norm = C63 / (C21_modify^3);

cumulants(row, 1) = abs(C20_norm);

cumulants(row, 2) = abs(C21_norm);

cumulants(row, 3) = abs(C40_norm);

cumulants(row, 4) = abs(C41_norm);

cumulants(row, 5) = abs(C42_norm);

cumulants(row, 6) = abs(C60_norm);

cumulants(row, 7) = abs(C61_norm);

cumulants(row, 8) = abs(C62_norm);

cumulants(row, 9) = abs(C63_norm);

end

实验结果

默认参数

相关参数

样本sample.dat 的md5：71538ce351a79374a5705b2f08ced2ac

| 高阶累积量 | 每组数据 | 样本集每种调制的样本数量 | 测试集每种调制的测试数据量 | KNN算法参数 |
| --- | --- | --- | --- | --- |
| 4阶 | N=500 | M=10 | P=1000 | 10 |

运行结果

|  | KNN | BPSK | QPSK | 16QAM |
| --- | --- | --- | --- | --- |
| 运行日志 |  | the total correct rate on -4 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on -2 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 0.0 判断为16QAM: 1.0 判断为64QAM: 999.0 the total correct rate on 0 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 0.0 判断为16QAM: 127.0 判断为64QAM: 873.0 the total correct rate on 2 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 25.0 判断为16QAM: 975.0 判断为64QAM: 0.0 the total correct rate on 4 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 6 dB SNR is: 0.893 正确判断为BPSK: 893.0 判断为QPSK: 107.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 8 dB SNR is: 1.0 正确判断为BPSK: 1000.0 判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 10 dB SNR is: 1.0 正确判断为BPSK: 1000.0 判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 0.0 OrderedDict({-4: 0.0, -2: 0.0, 0: 0.0, 2: 0.0, 4: 0.0, 6: 0.893, 8: 1.0, 10: 1.0}) | the total correct rate on -4 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on -2 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 0 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 2 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 4 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 4.0 判断为64QAM: 996.0 the total correct rate on 6 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 879.0 判断为64QAM: 121.0 the total correct rate on 8 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 1000.0 判断为64QAM: 0.0 the total correct rate on 10 dB SNR is: 0.441 判断为BPSK: 0.0 正确判断为QPSK: 441.0 判断为16QAM: 559.0 判断为64QAM: 0.0 OrderedDict({-4: 0.0, -2: 0.0, 0: 0.0, 2: 0.0, 4: 0.0, 6: 0.0, 8: 0.0, 10: 0.441}) | the total correct rate on 0 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 5 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 10 dB SNR is: 0.01 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 10.0 判断为64QAM: 990.0 the total correct rate on 15 dB SNR is: 0.693 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 693.0 判断为64QAM: 307.0 the total correct rate on 20 dB SNR is: 0.957 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 957.0 判断为64QAM: 43.0 the total correct rate on 25 dB SNR is: 0.99 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 990.0 判断为64QAM: 10.0 the total correct rate on 30 dB SNR is: 0.995 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 995.0 判断为64QAM: 5.0 the total correct rate on 35 dB SNR is: 0.995 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 995.0 判断为64QAM: 5.0 the total correct rate on 40 dB SNR is: 0.992 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 992.0 判断为64QAM: 8.0 OrderedDict({0: 0.0, 5: 0.0, 10: 0.01, 15: 0.693, 20: 0.957, 25: 0.99, 30: 0.995, 35: 0.995, 40: 0.992}) |
| 结果截图 |  |  |  |  |

参数N=200

相关参数

样本sample.dat 的md5：9abcaa4aa12b35f0beb16cea9fcb9b4b

| 高阶累积量 | 每组数据 | 样本集每种调制的样本数量 | 测试集每种调制的测试数据量 | KNN算法参数 |
| --- | --- | --- | --- | --- |
| 4阶 | N=200 | M=10 | P=1000 | 10 |

运行结果

|  | KNN | BPSK | QPSK | 16QAM |
| --- | --- | --- | --- | --- |
| 运行日志 |  | the total correct rate on -4 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 0.0 判断为16QAM: 74.0 判断为64QAM: 926.0 the total correct rate on -2 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 0.0 判断为16QAM: 105.0 判断为64QAM: 895.0 the total correct rate on 0 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 0.0 判断为16QAM: 339.0 判断为64QAM: 661.0 the total correct rate on 2 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 179.0 判断为16QAM: 722.0 判断为64QAM: 99.0 the total correct rate on 4 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 996.0 判断为16QAM: 4.0 判断为64QAM: 0.0 the total correct rate on 6 dB SNR is: 0.765 正确判断为BPSK: 765.0 判断为QPSK: 235.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 8 dB SNR is: 1.0 正确判断为BPSK: 1000.0 判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 10 dB SNR is: 1.0 正确判断为BPSK: 1000.0 判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 0.0 OrderedDict({-4: 0.0, -2: 0.0, 0: 0.0, 2: 0.0, 4: 0.0, 6: 0.765, 8: 1.0, 10: 1.0}) | the total correct rate on -4 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 24.0 判断为64QAM: 976.0 the total correct rate on -2 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 12.0 判断为64QAM: 988.0 the total correct rate on 0 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 21.0 判断为64QAM: 979.0 the total correct rate on 2 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 58.0 判断为64QAM: 942.0 the total correct rate on 4 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 348.0 判断为64QAM: 652.0 the total correct rate on 6 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 906.0 判断为64QAM: 94.0 the total correct rate on 8 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 1000.0 判断为64QAM: 0.0 the total correct rate on 10 dB SNR is: 0.338 判断为BPSK: 0.0 正确判断为QPSK: 338.0 判断为16QAM: 662.0 判断为64QAM: 0.0 OrderedDict({-4: 0.0, -2: 0.0, 0: 0.0, 2: 0.0, 4: 0.0, 6: 0.0, 8: 0.0, 10: 0.338}) | the total correct rate on 0 dB SNR is: 0.004 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 4.0 判断为64QAM: 996.0 the total correct rate on 5 dB SNR is: 0.011 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 11.0 判断为64QAM: 989.0 the total correct rate on 10 dB SNR is: 0.242 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 242.0 判断为64QAM: 758.0 the total correct rate on 15 dB SNR is: 0.649 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 649.0 判断为64QAM: 351.0 the total correct rate on 20 dB SNR is: 0.764 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 764.0 判断为64QAM: 236.0 the total correct rate on 25 dB SNR is: 0.832 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 832.0 判断为64QAM: 168.0 the total correct rate on 30 dB SNR is: 0.829 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 829.0 判断为64QAM: 171.0 the total correct rate on 35 dB SNR is: 0.833 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 833.0 判断为64QAM: 167.0 the total correct rate on 40 dB SNR is: 0.85 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 850.0 判断为64QAM: 150.0 OrderedDict({0: 0.004, 5: 0.011, 10: 0.242, 15: 0.649, 20: 0.764, 25: 0.832, 30: 0.829, 35: 0.833, 40: 0.85}) |
| 结果截图 |  |  |  |  |

变换K参数

K=3

样本sample.dat 的md5：71538ce351a79374a5705b2f08ced2ac

| 高阶累积量 | 每组数据 | 样本集每种调制的样本数量 | 测试集每种调制的测试数据量 | KNN算法参数 |
| --- | --- | --- | --- | --- |
| 4阶 | N=500 | M=10 | P=1000 | 3 |

运行结果

|  | KNN | BPSK | QPSK | 16QAM |
| --- | --- | --- | --- | --- |
| 运行日志 |  | the total correct rate on -4 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on -2 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 0 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 0.0 判断为16QAM: 2.0 判断为64QAM: 998.0 the total correct rate on 2 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 168.0 判断为16QAM: 826.0 判断为64QAM: 6.0 the total correct rate on 4 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 6 dB SNR is: 0.734 正确判断为BPSK: 734.0 判断为QPSK: 266.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 8 dB SNR is: 1.0 正确判断为BPSK: 1000.0 判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 10 dB SNR is: 1.0 正确判断为BPSK: 1000.0 判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 0.0 OrderedDict({-4: 0.0, -2: 0.0, 0: 0.0, 2: 0.0, 4: 0.0, 6: 0.734, 8: 1.0, 10: 1.0}) | the total correct rate on -4 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on -2 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 0 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 2 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 4 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 6 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 44.0 判断为64QAM: 956.0 the total correct rate on 8 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 998.0 判断为64QAM: 2.0 the total correct rate on 10 dB SNR is: 0.102 判断为BPSK: 0.0 正确判断为QPSK: 102.0 判断为16QAM: 898.0 判断为64QAM: 0.0 OrderedDict({-4: 0.0, -2: 0.0, 0: 0.0, 2: 0.0, 4: 0.0, 6: 0.0, 8: 0.0, 10: 0.102}) | the total correct rate on 0 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 5 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 10 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 15 dB SNR is: 0.134 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 134.0 判断为64QAM: 866.0 the total correct rate on 20 dB SNR is: 0.757 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 757.0 判断为64QAM: 243.0 the total correct rate on 25 dB SNR is: 0.902 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 902.0 判断为64QAM: 98.0 the total correct rate on 30 dB SNR is: 0.943 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 943.0 判断为64QAM: 57.0 the total correct rate on 35 dB SNR is: 0.957 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 957.0 判断为64QAM: 43.0 the total correct rate on 40 dB SNR is: 0.959 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 959.0 判断为64QAM: 41.0 OrderedDict({0: 0.0, 5: 0.0, 10: 0.0, 15: 0.134, 20: 0.757, 25: 0.902, 30: 0.943, 35: 0.957, 40: 0.959}) |
| 结果截图 |  |  |  |  |

K=35

样本sample.dat 的md5：71538ce351a79374a5705b2f08ced2ac

| 高阶累积量 | 每组数据 | 样本集每种调制的样本数量 | 测试集每种调制的测试数据量 | KNN算法参数 |
| --- | --- | --- | --- | --- |
| 4阶 | N=500 | M=10 | P=1000 | 35 |

运行结果

|  | KNN | BPSK | QPSK | 16QAM |
| --- | --- | --- | --- | --- |
| 运行日志 |  | the total correct rate on -4 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on -2 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 0 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 2 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 4 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 6 dB SNR is: 0.999 正确判断为BPSK: 999.0 判断为QPSK: 1.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 8 dB SNR is: 1.0 正确判断为BPSK: 1000.0 判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 10 dB SNR is: 1.0 正确判断为BPSK: 1000.0 判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 0.0 OrderedDict({-4: 0.0, -2: 0.0, 0: 0.0, 2: 0.0, 4: 0.0, 6: 0.999, 8: 1.0, 10: 1.0}) | the total correct rate on -4 dB SNR is: 1.0 判断为BPSK: 0.0 正确判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on -2 dB SNR is: 1.0 判断为BPSK: 0.0 正确判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 0 dB SNR is: 1.0 判断为BPSK: 0.0 正确判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 2 dB SNR is: 1.0 判断为BPSK: 0.0 正确判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 4 dB SNR is: 1.0 判断为BPSK: 0.0 正确判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 6 dB SNR is: 1.0 判断为BPSK: 0.0 正确判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 8 dB SNR is: 1.0 判断为BPSK: 0.0 正确判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 10 dB SNR is: 1.0 判断为BPSK: 0.0 正确判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 OrderedDict({-4: 1.0, -2: 1.0, 0: 1.0, 2: 1.0, 4: 1.0, 6: 1.0, 8: 1.0, 10: 1.0}) | the total correct rate on 0 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 1000.0 正确判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 5 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 1000.0 正确判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 10 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 1000.0 正确判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 15 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 1000.0 正确判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 20 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 1000.0 正确判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 25 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 1000.0 正确判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 30 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 1000.0 正确判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 35 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 1000.0 正确判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 40 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 1000.0 正确判断为16QAM: 0.0 判断为64QAM: 0.0 OrderedDict({0: 0.0, 5: 0.0, 10: 0.0, 15: 0.0, 20: 0.0, 25: 0.0, 30: 0.0, 35: 0.0, 40: 0.0}) |
| 结果截图 |  |  |  |  |

六阶-默认参数

相关参数

样本sample.dat 的md5：e6df415936d34f291f5161f737ce2ad5

| 高阶累积量 | 每组数据 | 样本集每种调制的样本数量 | 测试集每种调制的测试数据量 | KNN算法参数 |
| --- | --- | --- | --- | --- |
| 6阶 | N=500 | M=10 | P=1000 | 10 |

运行结果

|  | KNN | BPSK | QPSK | 16QAM |
| --- | --- | --- | --- | --- |
| 运行日志 |  | the total correct rate on -4 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 0.0 判断为16QAM: 5.0 判断为64QAM: 995.0 the total correct rate on -2 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 0.0 判断为16QAM: 86.0 判断为64QAM: 914.0 the total correct rate on 0 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 1.0 判断为16QAM: 961.0 判断为64QAM: 38.0 the total correct rate on 2 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 964.0 判断为16QAM: 36.0 判断为64QAM: 0.0 the total correct rate on 4 dB SNR is: 0.0 正确判断为BPSK: 0.0 判断为QPSK: 1000.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 6 dB SNR is: 0.005 正确判断为BPSK: 5.0 判断为QPSK: 995.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 8 dB SNR is: 1.0 正确判断为BPSK: 1000.0 判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 0.0 the total correct rate on 10 dB SNR is: 1.0 正确判断为BPSK: 1000.0 判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 0.0 OrderedDict({-4: 0.0, -2: 0.0, 0: 0.0, 2: 0.0, 4: 0.0, 6: 0.005, 8: 1.0, 10: 1.0}) | the total correct rate on -4 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 2.0 判断为64QAM: 998.0 the total correct rate on -2 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 2.0 判断为64QAM: 998.0 the total correct rate on 0 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 1.0 判断为64QAM: 999.0 the total correct rate on 2 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 4 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 4.0 判断为64QAM: 996.0 the total correct rate on 6 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 945.0 判断为64QAM: 55.0 the total correct rate on 8 dB SNR is: 0.0 判断为BPSK: 0.0 正确判断为QPSK: 0.0 判断为16QAM: 1000.0 判断为64QAM: 0.0 the total correct rate on 10 dB SNR is: 0.47 判断为BPSK: 0.0 正确判断为QPSK: 470.0 判断为16QAM: 530.0 判断为64QAM: 0.0 OrderedDict({-4: 0.0, -2: 0.0, 0: 0.0, 2: 0.0, 4: 0.0, 6: 0.0, 8: 0.0, 10: 0.47}) | the total correct rate on 0 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 5 dB SNR is: 0.0 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 0.0 判断为64QAM: 1000.0 the total correct rate on 10 dB SNR is: 0.014 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 14.0 判断为64QAM: 986.0 the total correct rate on 15 dB SNR is: 0.563 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 563.0 判断为64QAM: 437.0 the total correct rate on 20 dB SNR is: 0.891 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 891.0 判断为64QAM: 109.0 the total correct rate on 25 dB SNR is: 0.956 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 956.0 判断为64QAM: 44.0 the total correct rate on 30 dB SNR is: 0.965 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 965.0 判断为64QAM: 35.0 the total correct rate on 35 dB SNR is: 0.968 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 968.0 判断为64QAM: 32.0 the total correct rate on 40 dB SNR is: 0.975 判断为BPSK: 0.0 判断为QPSK: 0.0 正确判断为16QAM: 975.0 判断为64QAM: 25.0 OrderedDict({0: 0.0, 5: 0.0, 10: 0.014, 15: 0.563, 20: 0.891, 25: 0.956, 30: 0.965, 35: 0.968, 40: 0.975}) |
| 结果截图 |  |  |  |  |

实验总结

四阶结果分析

在四阶累积量下有如下现象

- BPSK在SNR为8dB附近时正确率就可以达到1；QPSK在SNR>10dB时正确率才能达到1；16QAM一般在30dB之后准确率才能达到1

- N=200和N=500时对结果影响不明显

- K=3时，准确率无明显差异；K=35时，高阶调制的准确率明显下降

- 由上面的内容可以总结出下面几点

- 调制阶数越高，正确率达到1需要的SNR越大

- 偏小的K在训练的时候倾向于过拟合（在N=200和N=500时以及K=3和K=10时差距不明显是因为出现了过拟合），偏大的K在训练的时候偏向于欠拟合（K=35时不能很好的区分）

六阶累积量下有下面的结论

在信噪比比较差的情况下有明显的提升，使用更高阶的累积量意味着更多的参考特征，使得KNN性能有所提升。

改进方向包括：尝试SVM、深度学习等其他分类算法；探索时域、频谱等更多类型的特征；研究多径衰落等复杂信道条件下的识别问题；优化kNN的搜索效率以适应实时应用需求。